Internet of Things Fundamentals

Subject Project

BS AI 6th Semester SP-25 (AIE-3079)

Date: 26-06-2025

Project Title: Smart Doorbell System

Group Name: 						Neural Syncs

Team Members:

| Members | Registration no | Name | Signature |
| --- | --- | --- | --- |
| Member-1 (Leader) | 22-NTU-CS-1356 | Muhammad Sheeraz |  |
| Member-2 | 22-NTU-CS-1339 | Ayesha Fatima |  |
| Member-3 | 22-NTU-CS-1376 | Sumaiya Shehzadi |  |

| Contributions in % of each Team Members for each component |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Member-1 | Member-2 | Member-3 |
| Distribution Components |  | Ayesha Fatima | Muhammad Sheeraz | Sumaiya Shehzadi |
| Coding | ESP32-cam |  | 100% |  |
|  | Python Coding | 50% |  | 50% |
| UI Design |  |  | 100% |  |
| Database |  | 100% |  |  |
| Cloud Integration |  |  |  | 100% |
| IoT Gateway |  |  | 100% |  |
| Edge Processing |  | Nil | Nil | Nil |
| Documentation |  |  | 100% |  |
| Presentation Design |  | 50% |  | 50% |

# Team-Based Evaluation (60 Marks)

| Criteria | Obtained Marks | Out of |
| --- | --- | --- |
| System Design & Architecture |  | 10 |
| Hardware Integration & Circuit Setup |  | 10 |
| IoT Gateway and Cloud Communication |  | 10 |
| Working Prototype Demonstration |  | 10 |
| Performance & Reliability Testing |  | 10 |
| Presentation |  | 10 |
| Total (Team-Based) |  | 60 |

# Individual-Based Evaluation (40 Marks per Member)

|  | Member 1 | Member 2 | Member 3 | Member 4 |
| --- | --- | --- | --- | --- |
| Criteria |  |  |  |  |
| Understanding of the Project & Role | /10 | /10 | /10 | /10 |
| Code Contribution and Explanation | /10 | /10 | /10 | /10 |
| Q/A VIVA | /10 | /10 | /10 | /10 |
| Documentation/Reporting & Communication | /10 | /10 | /10 | /10 |
| Total (Individual-Based) | /40 | /40 | /40 | /40 |
| Total Overall (60+40) | /100 | /100 | /100 | /100 |
| Weightage Lab Grade (50) |  |  |  |  |

Table of Contents

Team-Based Evaluation (60 Marks)	2

Individual-Based Evaluation (40 Marks per Member)	2

1. Abstract / Executive Summary	5

2. Introduction	5

Background and Motivation	5

Problem Statement	5

Project Goals	5

3. Methodology / System Design	5

3.1 Hardware Components	5

3.2 Software Design	6

ESP32-CAM Firmware (Arduino IDE)	6

Python Server (Flask)	6

Cloud Integration	6

Pseudocode	8

4. Implementation	8

1. Hardware Wiring	8

2. ESP32-CAM Setup	9

3. Python Server Setup	9

4. Firebase Setup	9

Code Snippets	9

ESP32-CAM (C++):	9

Python (Face Recognition and Feedback):	9

Challenges and Solutions	9

6. Results and Discussion	10

Output Screenshots	10

Performance Evaluation	12

Comparison with Objectives	12

6. Testing and Validation / Limitations	12

Test Cases	12

Known Limitations	12

7. Conclusion and Future Work	13

Summary of Achievements	13

Future Enhancements	13

8. Links	13

GitHub Repository Link	13

List of Figures

Figure 1.1: Raspberry Pi and Peripherals	6

Figure 2.1: System Architecture Diagram	7

Figure 3.1: Component Interaction Diagram	7

Figure 4.1: Process Sequence	8

Figure 5.1: wi-Fi Connection and automatic server finding	10

Figure 6.1: Image Sending and Server Response	10

Figure 7.1: Flask Server Status in System Startup Service	11

Figure 8.1: web GUI	11

Figure 9.1: Firebase Logs	12

# 1. Abstract / Executive Summary

This project is a Smart Doorbell System that integrates computer vision, embedded hardware, and cloud-based services. An ESP32-CAM module captures visitor images and sends them over Wi-Fi to a Flask-based Python server hosted locally (e.g., Raspberry Pi). The server performs face recognition, displays results on an OLED screen, sounds a buzzer for feedback, and logs events in Firebase for remote access. The system emphasizes real-time performance, modularity, and extensibility.

# 2. Introduction

## Background and Motivation

Conventional doorbells offer no intelligence or remote monitoring. As smart home trends rise, there's a demand for systems that can identify visitors, offer real-time alerts, and keep logs for security.

## Problem Statement

- No identification or logging capabilities in traditional doorbells

- Lack of remote access or alert system

- Manual monitoring is insecure and inefficient

## Project Goals

- Perform real-time face recognition at the door

- Provide immediate visual and audio feedback

- Log all events to the cloud

- Maintain modularity for future upgrades

# 3. Methodology / System Design

## 3.1 Hardware Components

- ESP32-CAM (AI Thinker): Captures images and handles Wi-Fi connectivity

- OLED Display (SSD1306): Displays visitor identity

- Buzzer: Distinct audio feedback for known and unknown faces

- Power Supply: Provides power to the Raspberry Pi and peripherals

- Wiring: Connects Raspberry Pi to OLED and buzzer

Figure 1.1: Raspberry Pi and Peripherals

## 3.2 Software Design

### ESP32-CAM Firmware (Arduino IDE)

- Written in C++

- Connects to Wi-Fi and server using mDNS

- Captures an image every 5 seconds

- Sends the image via HTTP POST to the Flask server

### Python Server (Flask)

- Receives images at the /upload endpoint

- Extracts face embeddings using face_recognition and OpenCV

- Uses a pre-trained SVM classifier to identify the person

- Triggers OLED and buzzer feedback via GPIO

- Logs data to Firebase

### Cloud Integration

- Uses Firebase Realtime Database

- Logs include visitor name, confidence score, timestamp, and image filename

Figure 2.1: System Architecture Diagram

Figure 3.1: Component Interaction Diagram

Figure 4.1: Process Sequence

### Pseudocode

ESP32-CAM:

Connect to WiFi

Capture image every 5 seconds

Send image to Flask server via HTTP POST

Flask Server:

Receive image

Extract face embedding

Predict identity with SVM

Trigger OLED and buzzer

Log result to Firebase

# 4. Implementation

## 1. Hardware Wiring

- Wire the ESP32-CAM as per the circuit diagram

- Connect OLED (I2C) and buzzer to designated GPIO pins

- Provide power via 5V supply

## 2. ESP32-CAM Setup

- Open fast_speed_updated.ino in the Arduino IDE

- Set WiFi SSID and password

- Upload the code to the ESP32-CAM

## 3. Python Server Setup

- Install dependencies listed in requirementsP.txt

- Run train_model.py to train the SVM model

- Start the server using python app.py

## 4. Firebase Setup

- Configure Firebase credentials in app.py

- Ensure Firebase Realtime Database is enabled

## Code Snippets

### ESP32-CAM (C++):

http.begin(client, serverURL);

http.addHeader("Content-Type", "application/octet-stream");

int httpResponseCode = http.POST(fb->buf, fb->len);

### Python (Face Recognition and Feedback):

embedding = get_face_embedding(temp_path)

probs = model.predict_proba([embedding])[0]

predicted_label = model.classes_[np.argmax(probs)]

show_label_on_oled(predicted_label)

buzz_for(predicted_label)

### Challenges and Solutions

- WiFi Instability: Implemented auto-reconnect and watchdog timer

- Recognition Accuracy: Used SVM with embeddings to improve prediction

- Peripheral Integration: Used Adafruit libraries for OLED and GPIO management

# 6. Results and Discussion

## Output Screenshots

Figure 5.1: wi-Fi Connection and automatic server finding

Figure 6.1: Image Sending and Server Response

Figure 7.1: Flask Server Status in System Startup Service

Figure 8.1: web GUI

Figure 9.1: Firebase Logs

## Performance Evaluation

- Recognition Accuracy: High for trained faces

- Latency: Response time is under 10 seconds

- Reliability: Automatically recovers from connection failures

## Comparison with Objectives

- Real-time recognition and logging are successfully achieved

- System feedback (visual and audio) works as expected

- Cloud integration enables remote monitoring

# 6. Testing and Validation / Limitations

## Test Cases

- Known Face: Correct name shown, expected buzzer tone, log created

- Unknown Face: Labelled as "Unknown", different buzzer tone, log created

- WiFi Drop: ESP32 reconnects and resumes function

## Known Limitations

- Can only recognize faces from the trained dataset

- Requires stable WiFi and power supply

- Feedback limited to OLED and buzzer; no mobile alerts implemented yet

# 7. Conclusion and Future Work

## Summary of Achievements

- Built a fully functioning smart doorbell with real-time face recognition

- Achieved robust image capture, processing, feedback, and cloud logging

- Demonstrated a modular and extensible design

## Future Enhancements

- Add mobile app notifications

- Include motion detection and automatic door lock control

- Improve model using deep learning or a larger dataset

- Migrate to a more scalable cloud platform like AWS or Azure

# 8. Links

## GitHub Repository Link

https://github.com/Sheeraz-07/SMART-DOORBELL-IOT-PROJECT.git